# Shipboard Technical Support

# Meteorological Acquisition System

## Quickstart

Quick Startup Procedure From a powered off state

- Ensure power is on to all Met sensors.

- Power on the MET computer.

- Observe that the  Windows operating system is properly booting up.

- After bootup the user will see the MetAcq control panel on the display.

- At this point you can do one of two things.

  - Do nothing, The program will start in 3 minutes.

  - Click on the green control labled [START]

Note that the acquisition program is set to start automatically upon bootup.

Quick Startup Procedure From the Windows desktop

1.   Ensure Power is on to all Met Sensors.

- Click on the MetAcq icon.

- The MetAcq control panel will appear.

4.   At this point you can do one of two things.

  - Do nothing, The program will start in 3 minutes.

  - Click on the green control labled [START]

Program Operation

- To change Display parameters, click on [Select] button.

  - The Screen selection window appears.

  - Click on Item, select parameter

  - Select Sensor number <0> = Sensor 1 <1> = Sensor 2

  - Select number of decimal places.

  - Click on <x> in upper right corner to exit window.

- To change Strip Chart parameters, Click on variable next to chart display.

  - Make selection

  - After selection, click on [Clear Plot] button.

Copying Data Files

Insert a blank formatted 100MB or 250MB zip disk in the zip drive.

Click on the COPY control button located on the acquisition program control panel.

Click OK when ready to copy files.

This will copy all files in the MET data directory onto the  ZIP disk.

## Program Termination

Click on the Red [STOP] control button.

The Program will then enter standy mode.  The control panel will then

Show a green [START] control. Under this control is another control

Labled [EXIT]. Click on this control, the program will then terminate.

- Note that once the program enters standby mode, it will automatically

- restart the acquistition program after 3 minutes. Be sure to exit the

- program entirely if it is not desired to acquire data.

7-Mar-02  CM